#include <LiquidCrystal_I2C.h>

#include <Keypad.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#define ONE_WIRE_BUS 2

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

const byte ROWS = 4;  //four rows

const byte COLS = 4;  //three columns

char keys[ROWS][COLS] = {

{ '1', '2', '3', 'A' },

{ '4', '5', '6', 'B' },

{ '7', '8', '9', 'C' },

{ '*', '0', '#', 'D' }

};

byte rowPins[ROWS] = { 0, A3, 3, 4 };  //connect to the row pinouts of the keypad

byte colPins[COLS] = { 5, 6, 7, 8 };   //connect to the column pinouts of the keypad

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

LiquidCrystal_I2C lcd(0x27, 20, 4);

#define LOWER_LIMIT_SWITCH 1

#define UPPER_LIMIT_SWITCH A0

#define LIFTER_PIN_A A1

#define LIFTER_PIN_B A2

#define PUMP 9

#define BUZZER 10

#define HEATER 11

#define SPINNER 12

enum class MODE {

SETTING,

BOILING_WATER,

WAITING_FOR_CHICKEN,

PLACING_CHICKEN,

DEFEATHERING

};

MODE current_mode = MODE::SETTING;

struct BuzzerControl {

int duration;              // how long each beep lasts (ms)

int interval;              // pause between beeps (ms)

bool state;                // current buzzer state (on/off)

unsigned long lastChange;  // last state change time

bool active;               // is the buzzer currently active?

};

BuzzerControl buzzer = { 0, 0, false, 0, false };

int targetTemperature = 0;

int durationSeconds = 0;

int defeatheringDuration = 0;

unsigned long startMillis = 0;

void setup() {

// put your setup code here, to run once:

pinMode(LOWER_LIMIT_SWITCH, INPUT_PULLUP);

pinMode(UPPER_LIMIT_SWITCH, INPUT_PULLUP);

pinMode(LIFTER_PIN_A, OUTPUT);

pinMode(LIFTER_PIN_B, OUTPUT);

pinMode(PUMP, OUTPUT);

pinMode(BUZZER, OUTPUT);

pinMode(HEATER, OUTPUT);

pinMode(SPINNER, OUTPUT);

lcd.init();

lcd.backlight();

sensors.begin();

lcd.print(F("Resetting...."));

}

void loop() {

// put your main code here, to run repeatedly:

char key = keypad.getKey();

if (key == 'C') {

stopBeep();

current_mode = MODE::SETTING;

digitalWrite(SPINNER, LOW);

digitalWrite(LIFTER_PIN_A, LOW);

digitalWrite(LIFTER_PIN_B, LOW);

digitalWrite(PUMP, LOW);

lcd.clear();

delay(1000);

lcd.print(F("Resetting...."));

}

if (current_mode == MODE::SETTING) {

while (digitalRead(LOWER_LIMIT_SWITCH)) {

digitalWrite(LIFTER_PIN_A, LOW);

digitalWrite(LIFTER_PIN_B, HIGH);

}

digitalWrite(LIFTER_PIN_A, LOW);

digitalWrite(LIFTER_PIN_B, LOW);

lcd.setCursor(0, 0);

lcd.print(F("System Ready      "));

lcd.setCursor(0, 1);

lcd.print(F("Press # to start"));

char key = keypad.getKey();

while (key != '#') {

key = keypad.getKey();

}

// set temperature and duration in seconds

setTemperatureAndDuration();

current_mode = MODE::BOILING_WATER;

}

if (current_mode == MODE::BOILING_WATER) {

digitalWrite(HEATER, HIGH);

sensors.requestTemperatures();

float tempC = sensors.getTempCByIndex(0);

lcd.setCursor(0, 3);

if (tempC != DEVICE_DISCONNECTED_C) {

lcd.print(F("Temp: "));

lcd.print(tempC);

lcd.print(F(" C   "));

} else {

lcd.print("Temp: Error    ");

}

if (tempC >= targetTemperature) {

digitalWrite(HEATER, LOW);

current_mode = MODE::WAITING_FOR_CHICKEN;

startBeep(500, 500);

lcd.clear();

}

}

if (current_mode == MODE::WAITING_FOR_CHICKEN) {

lcd.setCursor(0, 0);

lcd.print(F("Press # to"));

lcd.setCursor(0, 1);

lcd.print(F("continue"));

updateBuzzer();

char key = keypad.getKey();

if (key == '#') {

lcd.clear();

current_mode = MODE::PLACING_CHICKEN;

startMillis = millis();

lcd.print(F("Set duration:"));

lcd.print(durationSeconds);

lcd.print(F("s"));

stopBeep();

}

}

if (current_mode == MODE::PLACING_CHICKEN) {

int current_time = (millis() - startMillis) / 1000;

lcd.setCursor(0, 1);

lcd.print(F("Time:"));

lcd.print(current_time);

lcd.print(F("s   "));

if (current_time >= durationSeconds) {

while (digitalRead(UPPER_LIMIT_SWITCH)) {

digitalWrite(LIFTER_PIN_A, HIGH);

digitalWrite(LIFTER_PIN_B, LOW);

}

digitalWrite(LIFTER_PIN_A, LOW);

digitalWrite(LIFTER_PIN_B, LOW);

current_mode = MODE::DEFEATHERING;

startMillis = millis();

lcd.clear();

lcd.print(F("Set Time:"));

lcd.print(defeatheringDuration);

}

}

if (current_mode == MODE::DEFEATHERING) {

int current_time = (millis() - startMillis) / 1000;

digitalWrite(SPINNER, HIGH);

digitalWrite(PUMP, HIGH);

lcd.setCursor(0, 1);

lcd.print(F("Time:"));

lcd.print(current_time);

lcd.print(F("s   "));

if (current_time >= defeatheringDuration) {

current_mode = MODE::SETTING;

digitalWrite(SPINNER, LOW);

digitalWrite(PUMP, LOW);

lcd.clear();

}

}

}

void setTemperatureAndDuration() {

// ================== TEMPERATURE ===================

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Temp (C):");

lcd.setCursor(0, 1);

String tempStr = "";

while (true) {

char k = keypad.getKey();

if (k) {

if (k >= '0' && k <= '9') {

tempStr += k;

lcd.print(k);

} else if (k == 'A') {  // Confirm input

targetTemperature = tempStr.toInt();

break;

} else if (k == 'B' && tempStr.length() > 0) {  // Backspace

tempStr.remove(tempStr.length() - 1);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Temp (C):");

lcd.setCursor(0, 1);

lcd.print(tempStr);

} else if (k == '*') {  // Cancel

return;

}

}

}

// ================== DURATION ===================

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Duration(s):");

lcd.setCursor(0, 1);

String durStr = "";

while (true) {

char k = keypad.getKey();

if (k) {

if (k >= '0' && k <= '9') {

durStr += k;

lcd.print(k);

} else if (k == 'A') {  // Confirm input

durationSeconds = durStr.toInt();

break;

} else if (k == 'B' && durStr.length() > 0) {  // Backspace

durStr.remove(durStr.length() - 1);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Duration(s):");

lcd.setCursor(0, 1);

lcd.print(durStr);

} else if (k == '*') {  // Cancel

return;

}

}

}

// ================== DEFEATHERING DURATION ===================

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Defeat(s):");

lcd.setCursor(0, 1);

String defeatherStr = "";

while (true) {

char k = keypad.getKey();

if (k) {

if (k >= '0' && k <= '9') {

defeatherStr += k;

lcd.print(k);

} else if (k == 'A') {  // Confirm input

defeatheringDuration = defeatherStr.toInt();

break;

} else if (k == 'B' && defeatherStr.length() > 0) {  // Backspace

defeatherStr.remove(defeatherStr.length() - 1);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Set Defeat(s):");

lcd.setCursor(0, 1);

lcd.print(defeatherStr);

} else if (k == '*') {  // Cancel

return;

}

}

}

// ================== SUMMARY ===================

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(targetTemperature);

lcd.print(" C");

lcd.setCursor(0, 1);

lcd.print("Time: ");

lcd.print(durationSeconds);

lcd.print("s");

lcd.setCursor(0, 2);

lcd.print("Defeat: ");

lcd.print(defeatheringDuration);

lcd.print("s");

delay(2500);

}

void startBeep(int duration, int interval) {

buzzer.duration = duration;

buzzer.interval = interval;

buzzer.state = false;

buzzer.lastChange = millis();

buzzer.active = true;

}

void stopBeep() {

buzzer.active = false;

digitalWrite(BUZZER, LOW);

}

void updateBuzzer() {

if (!buzzer.active) return;

unsigned long now = millis();

if (buzzer.state) {

// Buzzer is ON

if (now - buzzer.lastChange >= (unsigned long)buzzer.duration) {

digitalWrite(BUZZER, LOW);

buzzer.state = false;

buzzer.lastChange = now;

}

} else {

// Buzzer is OFF

if (now - buzzer.lastChange >= (unsigned long)buzzer.interval) {

digitalWrite(BUZZER, HIGH);

buzzer.state = true;

buzzer.lastChange = now;

}

}

}